2026 Crown Vic Rules

ABSOLUTLEY NO GREY AREA!!!!

Cars: 1993 & Up Ford Crown Vic, Grand Marquis, Town Cars only 4.6 L SOHC only, Engines must remain stock, Computers must remain Stock< No Tuning of any kind. Transmissions must Remain Stock, No Tuning of any Kind.

Wheels: Stock Steel or Aluminum from Crown Victoria or Mercury Grand Marquis.

All 4 wheels must be same size & off set

1-inch lug nuts are mandatory

No wheel spacers

No offset wheels on 2003-up

17-inch Dodge Charger wheels allowed on 2002-older.

Roll Cage: Minimum 4-point cage with 4 door bars on Drivers side (No straight bars on driver’s side) 1 ¾ diameter .095 thickness. Cages can be installed by any welder but must pass track inspection before it can be raced. Cage must be welded to frame not floor pan.

EXAMPLE OF CAGE BELOW

Racing Seat with 5-point harness only (NO Stock Seats)

Door Plate: 18” X 48” ¼ thick steel plate, minimum Plate must be placed inside of car

Body:

- All Panels MUST be Stock

- Gutting only allowed on doors -NO GUTTING of any other panels

- All panels must stick with magnet.

- Plastic inner fender wells can be removed.

Core support/Fender support may be replaced with 1 ½” – 2” box tubing. Must be at least 9” from frame to top bar. No Kickers, no gussets or supports of any kind. Stock support must have stock bushing.

Suspension:

- No cutting, Heating or lowering of springs.

- No Altering of suspension or springs.

- No Spacers of any kind.

No  BENDING OR SLIDING OF BUSHINGS OR CONTROL ARMS.

*****NO GREY AREA!!!****

Exhaust:

- Pipes must go behind driver or out to the side.

Tires:

- NO Racing Tires or Directional Tires allowed

- No autocross tires

- No Snow Tires

ALL SEASON TIRES: 400 TREADWEAR or HIGHER.

REQUIRED TIRES SIZES:

- 235/55/17

- 225/60/16

- 215/70/15

**Goodyear RSA tires are permitted.

**No Tire treatment or softener of any kind, Subject to tire testing will be driver responsibility. If tested tire comes back illegal, Driver will be fined $150 and lose points and prize money for that event.

Sway Bars Links:

- Stock front and rear, must be same length, and must be operational

Air Filter:

- Unaltered airbox must have OE compatible air filter

Manifold:

- Exhaust Manifold and Intake Manifold must be unaltered and stock.

Bumpers:

- Stock bumpers only.

- No fabricated bumpers

- Bumper hoops must be welded to bumper only, not to frame, Bumper Hoop must not be welded to bumper cover bracing.

- OEM covers from any type of vehicle (Front and Rear bumper cover required at all times)

**New Crown Vic Racing Nose will be allowed, Stock Bumper behind must

remain.

Shocks & Spring: Stock or OEM replacements. No coil overs, No Adjustable, No Spring Helpers of any kind. (can cut 2- 2” holes above rear shocks to access top shock nut)

Posi Units- No aftermarket lockers, you may weld spider gears or use Factory Ford Limited Slip.

Factory Gear only- 2.73, 3.27, 3.55 & 3.73-Ratio only

Steering Column: Stock only, extension and quick Disconnect allowed.

Windshield: 3 Windshield bars with screen only. No plexiglass.

Caster & Camber: You may adjust as factory adjustments allow. Camber Bolts may be used.

Dash:

- Must be original, air bags must be removed

- All unnecessary and unused wires may be removed

- HVAC box may be removed

- Must have working OBDII connector and Dash Cluster.

- If Dash Pad is removed, no sheet metal, Aluminum or any other material is allowed.

- Absolutely no Decking of any kind.

Weight: No added weight

Window nets are highly recommended

Battery: can be relocated to passenger floor pan or behind driver only, but must be secured in box Fuel Kill Switch in rear of deck or behind drivers roll cage or kill all switch if battery is relocated

Pedals: Stock pedal, in stock location. Spacers on pedals are ok.

Mirrors: No mirrors. All must be removed.

NO: Air Ride

NO: Fuel cells, must run factory fuel tank

**NO GRAY AREA-IF IT IS NOT LISTED IT IS NOT ALLOWED**

**All rules are subject to officials’ decisions**

**In Attempt to keep this class fair, rules may be modified at any time**